Stormwater Pollution Prevention

Plan

City of Trenton

Mercer County

NJG0151157

Annual Review Date: May  , 2024

Stormwater Program Coordinator:

Dalia Ghobrial, Chief of Water Quality

Table of Contents

Form 1 – Team Members	3

Form 2 – Revision History	4

Form 3 – Public Announcements	5

Form 4 – Post-Construction Stormwater Management in New Development and Redevelopment	6

Form 5 – Ordinances	8

Form 6 – Street Sweeping	9

Form 7 – MS4 Infrastructure	10

Form 8 – Community-wide Measures	13

Form 9 – Municipal Maintenance Yards & Other Ancillary Operations	14

Form 10 – Training	20

Form 11 – MS4 Mapping	23

Form 12 – Watershed Improvement Plan	24

# Form 1 – Team Members

| Stormwater Program Coordinator (SPC) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name and Title |  | Dalia Ghobrial Chief of Water Quality& Environmental Specialist 4 |  |  |  |  |
| Phone | 609-989-3097 |  | Email | Dghobrial@trentonnj.org |  |  |
| Individual(s) Responsible for Major Development Project Stormwater Management Review |  |  |  |  |  |  |
| Name and Title |  | Wesley Bridges Director of Law– Department of Law |  |  |  |  |
| Phone | 609-989-3039 |  | Email | WBridges@Trentonnj.org |  |  |
| Name and Title |  | Scott T. Miccio Assistant City Attorney-Department of Law |  |  |  |  |
|  | 609-989-3039 |  | Email |  | SMiccio@Trentonnj.org |  |
| Other Municipal Stormwater Team Members |  |  |  |  |  |  |
| Name and Title |  | Sean Semple Director of Water and Sewer |  |  |  |  |
| Phone | 609-433-4749 |  | Email |  | SSemple@trentonnj.org |  |
| Name and Title |  | Leigh Jones General Superintendent |  |  |  |  |
| Phone | 609-422-7620 |  | Email |  | LJones@trentonnj.org |  |
| Name and Title |  | Edwin Walker Sewer Superintendent |  |  |  |  |
| Phone | 609-575-8114 |  | Email |  | EWalker@Trentonnj.org |  |
| Shared/Contracted Service Providers |  |  |  |  |  |  |
| Provider Name |  | Service Provided |  |  |  | Term of Service |

# Form 2 – Revision History

| Revision Date | Form # Changed | Reason for Revision (Updates to staff, policy, webpage, etc.) |
| --- | --- | --- |
| 5/8/2017 |  | Annual Report |
| 9/27/2018 |  | SPPP Update |
| 12/20/2018 |  | SPPP Update |
| 2/7/2019 |  | SPPP Update |
| 9/25/2020 |  | Submission-Follow up to NJDEP notification |
| 4/30/2024 |  | Annual Report |
|  |  | SPPP Update |

# Form 3 – Public Announcements

Part IV.B. and C.

- Provide the link to the dedicated stormwater webpage for your municipality.

www.trentonnj.org/165/Education

- List the name and title of person(s) responsible for stormwater webpage postings/updates.

- List the newspapers, social media outlets, websites, direct mailings (Email or postal), and other communication approaches typically used to inform/educate the public on stormwater program information and related events/activities.

The City of Trenton educates the public by sending out NJDEP stormwater/ordinance education materials via ….. The City of Trenton maintains a Facebook page. Education material is found on the www.trentonnj.org/165/Education for students and the public.

# Form 4 – Post-Construction Stormwater Management in New Development and Redevelopment

Part IV.E.

- How does the municipality define “major development”? If it is different from the definition in N.J.A.C. 7:8, explain the difference.

Copy definition from SWM ordinance.

- Is the municipality’s stormwater control ordinance (SCO) the same as or more stringent than NJDEP’s model SCO? If more stringent, explain the difference.

The City of Trenton’s SCO is the same as NJDEP’s model SCO.

- Describe the process for reviewing major development project applications for compliance with the SCO and Residential Site Improvement Standards (RSIS).

All applications for major development projects are submitted to the Planning Board, Engineer and Planner. The Engineer reviews the plans to make sure they are in compliance with the Stormwater Control Ordinance and Residential Site Improvement Standards. The Planning and Zoning Boards will not approve any applications that are not in compliance with the Stormwater Control Ordinance and the Residential Site Improvements Standards. Any City project or project on City property will comply with a plan similar to the Stormwater Management Plan and will address and ensure long term compliance and maintenance of the BMP's for that project. All work will conform to the City of Trenton Stormwater Management Plan, including the design standards and the City’s Stormwater Requirements.

- Does your municipality have a mitigation plan included in your Municipal Stormwater Management Plan and Stormwater Control Ordinance? Indicate the location of records of all variances granted.

Yes. City has not issued any variances or exemptions. Documents are located with City of Trenton Clerk City of Trenton 1502 Lamberton Road Trenton, NJ 08611

- Indicate the dates of each iteration of the township’s Stormwater Control Ordinance, starting with the initial adoption and including revisions.

7/7/2006 (adopted) 9/7/2006 (effective date) Ord #0664 4/8/2024 Adopted

- Indicate the dates of each iteration of the township’s Municipal Stormwater Management Plan, starting with the initial adoption and including revisions.

3/17/2006 (Adopted) 9/7/2006 (County review-approved-failed to review in 60 days) 8/25/2018 (County review approved) 9/27/2018 12/20/2018 2/7/2019

# Form 5 – Ordinances

Part IV.F.1.

| Ordinance | Date Adopted | Was the DEP model adopted without change? If not, explain how the municipality’s is more stringent. | Entity Responsible for Enforcement | Fees & Fines |
| --- | --- | --- | --- | --- |
| - Pet Waste | 4/08/2024 |  |  | $___ |
| - Wildlife Feeding | 4/08/2024 |  |  | $___ |
| - Litter Control | 4/08/2024 |  |  | $__ |
| - Improper Disposal of Waste | 4/08/2024 |  |  | $__ |
| - Yard Waste | 4/08/2024 |  |  | $___ |
| - Private Storm Drain Inlet Retrofitting | 4/08/2024 |  |  | $___ |
| - Illicit Connections | 4/08/2024 |  |  | $___ |
| - Privately-Owned Salt Storage | 4/08/2024 |  |  | $___ |
| - Tree Removal-Replacement | 4/08/2024 |  |  | $___ |
| List any additional stormwater-related ordinances the municipality has adopted that address issues beyond the scope of the MS4 permit. Include adoption date, entity responsible for enforcement, and related fees and fines. |  |  |  |  |
| Stormwater Control Ordinance adopted______ Ordinance No. |  |  |  |  |
| Indicate the location of records associated with ordinances and related violations and enforcement actions below. |  |  |  |  |
| Documents are located with City of Trenton Clerk City of Trenton 1502 Lamberton Road Trenton, NJ 08611 |  |  |  |  |

# Form 6 – Street Sweeping

Part IV.F.2.a.i. and ii.

- Provide a written description and/or attach a map outlining the sweeping schedule for the following: - Segments of municipal roads with storm drain inlets that discharge to surface water (required at least 3 times each year) - Segments of municipal roads that do not have storm drain inlets but do discharge to surface water (required at least 1 time each year) Note: Only asphalt and concrete roads need to be swept. Roads that do not have storm drain inlets and do not discharge to surface water do not need to be swept.

All required streets are swept (4,400 miles). Streets are swept Monday – Friday 8 am to 3 pm Street sweeping schedule is attached.

- Indicate if sweeping work is outsourced and if so, describe the arrangement.

No.

# Form 7 – MS4 Infrastructure

Part IV.F.2-4. and Part IV.G.2-3.

- Municipal Storm Drain Inlets - Describe how you ensure that municipal inlets without permanent wording cast into the design have been properly labelled. - Describe how you ensure that municipal and private storm drain inlets have been retrofitted. - Describe how you ensure that newly installed storm drain inlets include corresponding catch basins or other BMPs to collect solids. - Describe when and how you conduct inspections of storm drain inlets and the criteria used to determine when they need to be cleaned.

- Three thousand (3000) inlets are inspected, and labels are replaced as needed. Inlet heads are also replaced within the project area of City projects. - For any repaving, repairing, reconstruction, or alternation project on any City road that contains inlets that do not meet the current standard, the inlet chambers, frames, grates, and/or inlet curb pieces are replaced as part of the project as necessary. For any project requiring privately owned storm drains to be retrofitted, the City Engineer inspects the site to make sure the inlets are properly retrofitted prior to issuing final approval and/or Certificate of Occupancy at the end of the project. - During design and review, the City Engineer will ensure the proposed design includes catch basins or other BMPs to collect solids. During construction, the City Engineer will ensure the proper catch basins or other BMPs meant to collect solids will be installed. - Inlets are visually inspected and cleaned daily by the Street Department. Records are kept on file. Maintenance is scheduled for those catch basins that are in disrepair as needed.

- Municipal Catch Basins - Describe when and how you conduct inspections of catch basins. - Describe the criteria used to determine when catch basins need to be cleaned.

a. The catch basins in the city are on a rotating cleaning schedule that operates daily every week. Catch basins are visually inspected at that time. b. Catch basins are cleaned during street sweeping. The catch basins are visually inspected for: buildup of brush, leaves, and trash; soil erosion; structural deficiencies; backup of water; etc. If these problems are found, the catch basin is then scheduled for repair.

- Municipal Conveyance System Describe when and how inspections of MS4 conveyance systems are conducted, and the criteria used to determine when they need to be cleaned. Include a description of the equipment and techniques used.

The City’s conveyance system is visually inspected on an annual basis and inspected again if problems arise. Observations such as flooded inlets may indicate clogged pipes or back-ups in the conveyance system. The Department of Public Works will investigate the area of concern, determine the problem(s), and take appropriate measures to fix the issue(s). If the problem cannot be fixed in-house, such as in cases where more attention and equipment is required, then the repair work will be outsourced to a contractor. An example of work outsourced is videoing and jetting of pipes.

- Municipal Outfall Inspections – Stream Scouring Describe the program in place to detect, investigate, and control localized stream scouring from stormwater outfalls. Include a description of the equipment and techniques used.

The MS4 Stormwater Outfall Map, which identifies the location of all outfall pipes operated by the City (141) and Private ownership (9). The City visually inspects each outfall yearly and schedules repairs for any stream scouring.

- Municipal Outfall Inspections – Illicit Discharge Detection and Elimination Describe the program in place for conducting visual dry weather inspections of municipally owned or operated outfalls. Include a description of the equipment and techniques used. Record cases of illicit discharges using the DEP’s Illicit Connection Inspection Report Form from the Department’s main stormwater webpage.

The City has conducted an initial physical inspection of all outfall pipes (141 City & 9 Private) during the mapping process. The City will use the NJDEP Illicit Connection Inspection Report Form to conduct these inspections, and each of these forms will be kept with the other SPPP reports. Outfall pipes that are found to have a dry weather flow or evidence of an intermittent non-stormwater flow will be further evaluated to locate potential illicit connections. If the City locates any illicit connections, the City will cite the responsible party for being in violation of the Illicit Connection Ordinance and will take actions to eliminate the illicit connection(s). If after a reasonable level of investigation, the City is unable to locate the source of the illicit connection, the City will submit the Closeout Investigation Form with the City's Annual Inspection and Recertification. If an illicit connection is found to originate from another public entity, the City will report the illicit connection to the Department. The City has developed and adopted an Illicit Connection Elimination Program in 2018 and updated 4/8/2024.

- Other Municipal Infrastructure List the types of MS4 infrastructure in your town that require inspection but are not noted above in items 1-5. Describe when and how you conduct inspections of this infrastructure and the criteria used to determine when they need to be maintained and/or cleaned.

N/A

- Stormwater Facilities Not Owned or Operated by the Municipality Describe your program for ensuring adequate long-term cleaning, operation, and maintenance of stormwater facilities not owned or operated by the municipality. This should include your plan for ensuring annual inspections are being done on these private properties and describe how you record the locations and logs associated with private infrastructure.

All privately owned stormwater facilities on commercial and private sites are inspected annually, and the owners are required to make necessary repairs.

- Infrastructure Records Indicate the location of records related to stormwater infrastructure inspection, cleaning, maintenance, and repair activities.

Documents are located with: City of Trenton Clerk 1502 Lamberton Road and the Public Works Facility 333 Cortland Street

# Form 8 – Community-wide Measures

Part IV.F.2.

- Herbicide Application Management Describe your program for preventing herbicides from being washed into the waters of the State and to prevent erosion caused by de-vegetation.

No Herbicides.

- Excess Deicing Material Management Describe your program for ensuring that excess salt piles are removed in a timely manner after storm events.

A salt dome building is located on the Division of Streets yard. The condition is fine and in compliance with the Stormwater Permit. During inspection, a stockpile of salt was located on an impervious surface and covered with tarps. However, the tarps were held down with old tires. The tires must be properly removed, and the tarps must be held down with a method that does not contribute to stormwater runoff.

- Roadside Vegetative Waste Describe your program for ensuring proper pickup, handling, storage, and disposal of wood waste and yard trimmings generated by the permittee along municipal roads or on municipal properties (trimming trees, mowing, etc.).

- Roadside Erosion Control Describe your program to detect and repair erosion along municipal roadways.

# Form 9 – Municipal Maintenance Yards & Other Ancillary Operations

Part IV.F.5.

Please complete a separate Form 9 for each yard or site. Indicate the number of yards/sites the municipality owns or operates:  11

| - Site Name and Address |  |
| --- | --- |
| Public Works Facility – 333 Cortland Street Sewage Treatment Plant – Lamberton Road Division of Streets Yard -476 Brunswick Avenue Division of Solid Waste - 707 Calhoun Street Water Treatment Plant – Route 29 Property Maintenance Facility - 410 West Hanover Street Traffic & Signs Facility - 250 North Alley Police Headquarters – 225 North Clinton Avenue Fire Department - 244 Perry Street City Hall - 319 East State Street Auction Car Lot - Adjacent to 476 Brunswick Avenue |  |
| - Monthly Site Inspections Describe the nature of inspections conducted at this site and the location of inspection logs. |  |
| Field inspection of sites and noting any non-compliance. Report provided to Stormwater Program Coordinator & Chief of Water Quality detailing observations and providing solutions for compliance. Field date and yard inspected: 1/22/2024 Public Works Facility Sewage Treatment Plant Water Treatment Plant 2/6/2024 Division of Streets Yard Police Headquarters City Hall 2/8/2024 Division of Solid Waste Property Maintenance Facility Traffic & Signs Facility Fire Department Auction Car Lot |  |
| - Inventory List List all materials and machinery that are potentially exposed to stormwater. |  |
| Materials | Machinery/Equipment |
| See attachments | Auction yard-abandoned passenger cars, SUV’s, trucks |
|  | (panel, maintenance, and transport) |
|  | mowers and chippers. |
|  | Public Works-Abandoned Trucks (Flat bed, |
|  | pickups, and panel) |
|  | Division of Solid Waste-misc. rusted equipm’t |
|  | Traffic & Signs Facility-Abandoned Forklift, |
|  | snowplow, misc. equipment. |
|  | Fire Department-few cars |
| - Discharge of Stormwater from Secondary Containment Describe the process in place for discharging stormwater from secondary containment areas where outdoor containers are stored. |  |
| What containment is used? Any secondary containment at any of the 11 yards? |  |
| - Fueling Operations Does fueling occur on site? If so, describe the BMPs in place to minimize contamination of stormwater from fueling activities. If not, explain where fueling takes place. |  |
| Division of Solid Waste - The vehicle fueling facility onsite is not sufficient. - Drip pans will be placed under hose and pipe connections and other leak prone areas. This facility needs to be upgraded. Fire Department - Diesel tanks and drums are stored in an enclosed area just outside of the garage. The tanks are single wall and do not meet the requirements of the stormwater permit. Drums and tanks are be double-walled tanks or in a containment chamber large enough to hold 110% of the contents. |  |
| - Vehicle/Equipment Maintenance and Repair Do you perform maintenance and repair on site? Is this conducted indoors or outdoors? If outdoors, describe the BMPs in place to minimize contamination of stormwater from maintenance and repair activities. |  |
| Public Works Facility – Example: maintenance on-site, indoors, floor drains to degreaser, spills contained (towels, kitty litter, etc.) and disposed of properly. Fluids collected and disposed at proper facility (name). Sewage Treatment Plant – Division of Streets Yard – Division of Solid Waste – Water Treatment Plant – Property Maintenance Facility – Traffic & Signs Facility – Police Headquarters – Fire Department – City Hall – No Auction Car Lot - . |  |
| - Wash Wastewater Containment Do you wash vehicles on site? If so, describe the BMPs in place to minimize contamination of stormwater from these activities. Note that on site containment structures require annual inspections by a NJ licensed professional engineer. If not, explain where vehicle washing takes place. |  |
| Vehicles are not washed on-site. Vehicles are washed at….. |  |
| - Salt and Other Granular De-icing Materials Do you store salt and other granular deicing materials on site? If so, describe how they are stored and the BMPs in place to minimize contamination of stormwater from these materials. If not, explain where these materials are stored. |  |
| Division of Streets - A salt dome building is in fine condition and in compliance with the Stormwater Permit. There was a stockpile of salt located on an impervious surface and covered with tarps. However, the tarps were held down with old tires. If remaining, the tires must be properly removed, and the tarps must be held down with a method that does not contribute to stormwater runoff. A large calcium chloride tank with multiple hoses lying on the ground is in the yard. The tank must be a double walled tank, or containment must be provided in the amount of 110% of the volume of the calcium chloride tank. Please indicate if the materials are now properly stored and revise the note. |  |
| - Aggregate Material, Wood Chips, and Finished Leaf Compost Do you store these materials on site? If so, describe how they are stored and the BMPs in place to minimize contamination of stormwater from these materials. If not, explain where these materials are stored. |  |
| Material is directly hauled to a facility (name)? |  |
| - Cold Patch Asphalt Do you store these materials on site? If so, describe how they are stored and the BMPs in place to minimize contamination of stormwater from these materials. If not, explain where these materials are stored. |  |
| Division of Streets, Division of Public Works The cold patch storage area is open to the weather, not adequately covered and much of the cold patch is open to rainwater. Cold patch is to be stored in a permanent structure or on an impervious surface and covered with a waterproof material and contained to control leachate. Please indicate if the materials are now properly stored and revise the note. |  |
| - Street Sweepings and Storm Sewer Cleanout Materials Do you store these materials on site? If so, describe how they are stored and the BMPs in place to minimize contamination of stormwater from these materials. If not, explain where these materials are stored. |  |
| 2,000 tons of material were swept from 4,400 miles of streets. 435 cy of debris was removed from 3000 catch basins. Material is directly hauled to a facility (name)? |  |
| - Construction and Demolition Waste, Wood Waste, and Yard Trimmings Do you store these materials on site? If so, describe how they are stored and the BMPs in place to minimize contamination of stormwater from these materials. If not, explain where these materials are stored. |  |
| Material is directly hauled to a facility (name)? |  |
| - Scrap Tires Do you store these materials on site? If so, describe how they are stored and the BMPs in place to minimize contamination of stormwater from these materials. If not, explain where these materials are stored. |  |
| Please see Questionnaire section Municipal Maintenance Yard…Operations #2 Sewage Treatment Plant – Piles of scrap tires stored in the open. To be in waterproof area or removed. Division of Streets Yard – Piles of scrap tires stored in the open. To be in waterproof area or removed. Division of Solid waste - Piles of scrap tires stored in the open. To be in waterproof area or removed. Property Maintenance Facility - Piles of scrap tires stored in the open. To be in waterproof area or removed. Traffic & Signs Facility - Scrap tire stored in the open. To be in waterproof area or removed. Auction Car Lot – Piles of scrap tires stored in the open. To be in waterproof area or removed. |  |
| - Inoperable Vehicles and Equipment Do you store inoperable vehicles or equipment on site? If so, describe how they are stored and the BMPs in place to minimize contamination of stormwater. If not, explain where they are stored. |  |
| Public Works Facility – – Storage of inoperable cars. To be covered from the rain. Fluids removed. Division of Streets Yard - Storage of inoperable cars and equipment-open. To be covered from the rain. Fluids removed. Division of Solid Waste - Storage of inoperable cars and equipment-open. To be covered from the rain. Fluids removed. Traffic & Signs Facility - Inoperable equipment-open. To be covered from the rain. Fluids removed. Fire Department - Storage of inoperable cars and equipment-open. To be covered from the rain. Fluids removed. Auction Car Lot – Storage of inoperable cars. To be covered from the rain. Fluids removed. |  |

Form 10 – Training

Part IV.F.6-10.

Stormwater Program Coordinators

Describe the training provided for the municipal Stormwater Program Coordinator.

Online and/or in-person training is required for the Stormwater Program Coordinator. The Stormwater Program Coordinator conducts yearly training and any training required by the NJDEP as regulations are amended and updated.

| Topic | Municipal Employees Examples: in-person or virtual group sessions, e-Learning, field trainings, and videos |
| --- | --- |
| Describe the training provided for municipal staff. |  |
| SPPP | City of Trenton staff complete training within 3 months of being hired and on a yearly basis afterward. Staff watch the stormwater training videos provided by the NJDEP. |
| Construction Site Stormwater Runoff | City of Trenton staff complete training within 3 months of being hired and on a yearly basis afterward. Staff watch the stormwater training videos provided by the NJDEP. |
| Post-Construction Stormwater Management in New and Redevelopment | City of Trenton staff complete training within 3 months of being hired and on a yearly basis afterward. Staff watch the stormwater training videos provided by the NJDEP. |
| Community-wide Ordinances | City of Trenton staff are active and/or made aware of new ordinances and regulations. |
| Community-wide Measures | The city administrator plans all stormwater community events/measures and sends notifications to the city staff as well as residents. |
| Stormwater Facilities Maintenance | City of Trenton staff complete training within 3 months of being hired and on a yearly basis afterward. Staff watch the stormwater training videos provided by the NJDEP. The Department of Public Works will complete the yearly training as required by the NJDEP. |
| Municipal Maintenance Yards and Other Ancillary Operations | Designated staff at the following facilities will complete the yearly training as required by the NJDEP: Public Works Facility Sewage Treatment Plant Division of Streets Yard Division of Solod Waste Water Treatment Plant Property Maintenance Facility Traffic & Signs Facility Police Department Fire Department City Hall Auction Car Lot |
| MS4 Mapping | City of Trenton staff are provided with copies of the completed map and are made aware of any changes that are made to the stormwater map. Map noted in the Questionnaire. |
| Outfall Stream Scouring | The Department of Public Works is trained to recognize and repair stream scouring. The DPW will complete the yearly training as required by the NJDEP. |
| Illicit Discharge Detection and Elimination | The Department of Public Works is trained to recognize and investigate discharges. The DPW will complete the yearly training as required by the NJDEP. |

Stormwater Management Design Reviewers

Describe the training provided for individuals responsible for reviews and approvals of stormwater management designs.

They complete the 5-year NJDEP stormwater training as well as training required at any amendments to the stormwater regulations.

Municipal Board and Governing Body Members

Describe the training provided for members of the planning/zoning board and municipal council.

Members of the Planning and Zoning Boards and City Council watch the training videos provided by NJDEP.

Training Records

Indicate the location of training records for the above required training.

Documents are located with City of Trenton Clerk City of Trenton 1502 Lamberton Road Trenton, NJ 08611

# Form 11 – MS4 Mapping

Part IV.G.1.

| - Provide a link to the most current MS4 outfall/infrastructure map. |  |
| --- | --- |
| www.trentonnj.org/165/Education |  |
| - Indicate the total of each type of MS4 infrastructure listed below (due 01 Jan 2026). |  |
| - MS4 outfalls | 141 |
| - MS4 ground water discharge points (basins or overland flow infiltration areas) |  |
| - MS4 interconnections |  |
| - MS4 storm drain inlets |  |
| - MS4 manholes |  |
| - Length of conveyance (channels, pipes, ditches, etc.) |  |
| - MS4 pump stations |  |
| - MS4 stormwater facilities (any that are not listed above) |  |
| - Maintenance yard(s) and other ancillary operations | 11 |
| - Describe how the municipality’s outfall/infrastructure map is reviewed and updated to reflect any new or newly identified MS4 infrastructure (e.g., an outfall is closed, a new basin is constructed, ownership of an outfall has changed, etc.). |  |
| The map will be updated periodically to include new construction, maintenance, and repairs, etc. if required. |  |
| - Describe how the municipality will create and update its MS4 Infrastructure Map. |  |
| The existing map is reviewed annually and updated as needed. |  |

# Form 12 – Watershed Improvement Plan

Part IV.H.

- Describe how your municipality is developing its Watershed Improvement Plan.

- Describe any regional projects or collaboration efforts with other municipalities.

- Indicate the location of records related to all public information sessions and meetings for discussions of the Watershed Improvement Plan.